PXELINUX

A bootloader for Linux using the PXE network booting protocol

Copyright (C) 1994-2001 H. Peter Anvin

This program is provided under the terms of the GNU General Public

License, version 2 or, at your option, any later version.  There is no

warranty, neither expressed nor implied, to the function of this

program.  Please see the included file COPYING for details.

----------------------------------------------------------------------

PXELINUX is a SYSLINUX derivative, for booting Linux off a network

server, using a network ROM conforming to the Intel PXE (Pre-Execution

Environment) specification.  PXELINUX is *not* a program that is

intended to be flashed or burned into a PROM on the network card; if

you want that, check out NILO (http://www.nilo.org/).  NILO can also

be used to create a PXE-compliant boot PROM for most network cards

which have drivers for Linux or FreeBSD.

++++ HOW TO CONFIGURE PXELINUX ++++

PXELINUX operates in many ways like SYSLINUX.  If you are not familiar

with SYSLINUX, read syslinux.doc first, since this documentation only

explains the differences.

On the TFTP server, create the directory "/tftpboot", and copy the

following files to it:

pxelinux.0		- from the SYSLINUX distribution

any kernel or initrd images you want to boot

Finally, create the directory "/tftpboot/pxelinux.cfg".  The

configuration file (equivalent of syslinux.cfg -- see syslinux.doc for

the options here) will live in this directory.  Because more than one

system may be booted from the same server, the configuration file name

depends on the IP address of the booting machine.  PXELINUX will

search for its config file on the boot server in the following way:

First, it will search for the config file using its own IP address

in upper case hexadecimal, e.g. 192.0.2.91 -> C000025B

(you can use the included progam "gethostip" to compute the

hexadecimal IP address for any host.)

If that file is not found, it will remove one hex digit and try

again.  Ultimately, it will try looking for a file named "default"

(in lower case).

As an example, for 192.0.2.91, it will try C000025B, C000025,

C00002, C0000, C000, C00, C0, C, and default, in that order.

(See also the section on special DHCP options.)

It should be noted that all filename references are relative to the

directory pxelinux.0 lives in (usually /tftpboot).  PXELINUX

generally requires that filenames (including any relative path) are 63

characters or shorter in length.

PXELINUX does not support MTFTP, and I have no immediate plans of

doing so.  It is of course possible to use MTFTP for the initial boot,

if you have such a setup.  MTFTP server setup is beyond the scope of

this document.

++++ SETTING UP THE TFTP SERVER ++++

PXELINUX currently requires that the boot server has a TFTP server

which supports the "tsize" TFTP option (RFC 1784/RFC 2349).  The

"tftp-hpa" TFTP server, which support options, is available at:

http://www.kernel.org/pub/software/network/tftp/

ftp://www.kernel.org/pub/software/network/tftp/

... and on any kernel.org mirror (see http://www.kernel.org/mirrors/).

Another TFTP server which supports this is atftp by Jean-Pierre

Lefebvre:

ftp://ftp.mamalinux.com/pub/atftp/

Unfortunately, the Intel LANDesk Service Agent II prior to version

0.99h (PXE PDK V2.4) seems to have a host of serious bugs.  Among

other things, it requests the TFTP "blksize" option, but will be

mortally confused if this option is actually accepted by the server!

There are three possible workarounds for this bug:

1. Use a TFTP server with doesn't support "blksize".

Unfortunately, PXELINUX requires the "tsize" option to be

supported, and it is very unusual for TFTP servers to implement one

and not the other.  Both the tftp-hpa and atftp TFTP servers (see

above) therefore can be configured to disable the blksize option.

Use "-r blksize" for tftp-hpa, "--no-blksize" for atftp.

2. Use MTFTP for the initial bootstrap.  You need an MTFTP server with

the appropriate DHCP setup to do this.

3. Use appropriate DHCP options that the client will attempt MTFTP

before trying conventional TFTP.  It seems that the client will not

request the "blksize" option if it has tried MTFTP and failed.

These DHCP options are beyond the scope of this document.

These systems also seem to break if PMTU discovery is enabled on the

TFTP server.  This has to be disabled in the kernel on the TFTP

server.  On Linux, you can disable PMTU discovery by setting:

echo 1 > /proc/sys/net/ipv4/ip_no_pmtu_disc

Note that this may cause bad performance for other services on the machine.

++++ SETTING UP THE DHCP SERVER ++++

The PXE protocol uses a very complex set of extensions to DHCP or

BOOTP.  However, most PXE implementations -- this includes all Intel

ones version 0.99n and later -- seem to be able to boot in a

"conventional" DHCP/TFTP configuration.  Assuming you don't have to

support any very old or otherwise severely broken clients, this is

probably the best configuration unless you already have a PXE boot

server on your network.

A sample DHCP setup, using the "conventional TFTP" configuration,

would look something like the following, using ISC dhcp 2.0 dhcpd.conf

syntax:

allow booting;

allow bootp;

# Standard configuration directives...

option domain-name "<domain name>";

option subnet-mask <subnet mask>;

option broadcast-address <broadcast address>;

option domain-name-servers <dns servers>;

option routers <default router>;

# Group the PXE bootable hosts together

group {

# PXE-specific configuration directives...

next-server <TFTP server address>;

filename "/tftpboot/pxelinux.0";

# You need an entry like this for every host

# unless you're using dynamic addresses

host <hostname> {

hardware ethernet <ethernet address>;

fixed-address <hostname>;

}

}

Note that if your particular TFTP daemon runs under chroot (tftp-hpa

will do this if you specify the -s (secure) option; this is highly

recommended), you almost certainly should not include the /tftpboot

prefix in the filename statement.

If this does not work for your configuration, you probably should set

up a "PXE boot server" on port 4011 of your TFTP server; a free PXE

boot server is available at:

http://www.kano.org.uk/projects/pxe/

With such a boot server defined, your DHCP configuration should look

the same except for an "option dhcp-class-identifier":

allow booting;

allow bootp;

# Standard configuration directives...

option domain-name "<domain name>";

option subnet-mask <subnet mask>;

option broadcast-address <broadcast address>;

option domain-name-servers <dns servers>;

option routers <default router>;

# Group the PXE bootable hosts together

group {

# PXE-specific configuration directives...

option dhcp-class-identifier "PXEClient";

next-server <pxe boot server address>;

# You need an entry like this for every host

# unless you're using dynamic addresses

host <hostname> {

hardware ethernet <ethernet address>;

fixed-address <hostname>;

}

}

Here, the boot file name is obtained from the PXE server.

If the "conventional TFTP" configuration doesn't work on your clients,

and setting up a PXE boot server is not an option, you can attempt the

following configuration.  It has been known to boot some

configurations correctly; however, there are no guarantees:

allow booting;

allow bootp;

# Standard configuration directives...

option domain-name "<domain name>";

option subnet-mask <subnet mask>;

option broadcast-address <broadcast address>;

option domain-name-servers <dns servers>;

option routers <default router>;

# Group the PXE bootable hosts together

group {

# PXE-specific configuration directives...

option dhcp-class-identifier "PXEClient";

option vendor-encapsulated-options 09:0f:80:00:0c:4e:65:74:77:6f:72:6b:20:62:6f:6f:74:0a:07:00:50:72:6f:6d:70:74:06:01:02:08:03:80:00:00:47:04:80:00:00:00:ff;

next-server <TFTP server>;

filename "/tftpboot/pxelinux.0";

# You need an entry like this for every host

# unless you're using dynamic addresses

host <hostname> {

hardware ethernet <ethernet address>;

fixed-address <hostname>;

}

}

Note that this *will not* boot some clients that *will* boot with the

"conventional TFTP" configuration; Intel Boot Client 3.0 and later are

known to fall into this category.

++++ SPECIAL DHCP OPTIONS ++++

PXELINUX (starting with version 1.62) supports the following

nonstandard DHCP options, which depending on your DHCP server you may

be able to use to customize the specific behaviour of PXELINUX:

Option 208	pxelinux.magic

- Must be set to F1:00:74:7E (241.0.116.126) for PXELINUX to

recognize any special DHCP options whatsoever.

Option 209	pxelinux.configfile

- Specifies the PXELINUX configuration file name.

Option 210	pxelinux.pathprefix

- Specifies the PXELINUX common path prefix, instead of

deriving it from the boot file name.  This almost certainly

needs to end in whatever character the TFTP server OS uses

as a pathname separator, e.g. slash (/) for Unix.

Option 211	pxelinux.reboottime

- Specifies, in seconds, the time to wait before reboot in the

event of TFTP failure.  0 means wait "forever" (in reality,

it waits approximately 136 years.)

ISC dhcp 3.0 supports a rather nice syntax for specifying custom

options; you can use the following syntax in dhcpd.conf if you are

running this version of dhcpd:

option space pxelinux;

option pxelinux.magic      code 208 = string;

option pxelinux.configfile code 209 = text;

option pxelinux.pathprefix code 210 = text;

option pxelinux.reboottime code 211 = unsigned integer 32;

Then, inside your PXELINUX-booting group or class (whereever you have

the PXELINUX-related options, such as the filename option), you can

add, for example:

site-option-space pxelinux;

option pxelinux.magic f1:00:74:7e;

option pxelinux.configfile "configs/common";

option pxelinux.pathprefix "/tftpboot/pxelinux/files/";

option pxelinux.reboottime 30;

option dhcp-parameter-request-list 1, 3, 208, 209, 210, 211;

filename "/tftpboot/pxelinux/pxelinux.bin";

Note that the configfile is relative to the pathprefix: this will look

for a config file called /tftpboot/pxelinux/files/configs/common on

the TFTP server.

The "option dhcp-parameter-request-list" is unfortunately required,

since the DHCP exchange happens before PXELINUX starts.

Unfortunately at least DHCP 3.0rc2pl1 takes this as "send these

options and no others", even when the client explicitly requests

them.  This can easily cause problems for your operating system; you

may need to add additional options here.  I'm afraid I don't have a

neat and clean solution for this problem using existing software.

Fixing it in PXELINUX would require a fairly large amount of

additional code, since PXELINUX would be required to conduct its own

DHCP exchange rather than getting the cached packet from the PXE

stack.

Using ISC dhcp 3.0 you can create a lot of these strings on the fly.

For example, to use the hexadecimal form of the hardware address as

the configuration file name, you could do something like:

site-option-space pxelinux;

option pxelinux.magic f1:00:74:7e;

option pxelinux.configfile =

concat("pxelinux.cfg/", binary-to-ascii(16, 8, ":", hardware));

filename "/tftpboot/pxelinux.bin";

If you used this from a client whose Ethernet address was

58:FA:84:CF:55:0E, this would look for a configuration file named

"/tftpboot/pxelinux.cfg/1:58:fa:84:cf:55:e".

++++ SOME NOTES ++++

If the boot fails, PXELINUX (unlike SYSLINUX) will not wait forever;

rather, if it has not received any input for approximately five

minutes after displaying an error message, it will reset the machine.

This allows an unattended machine to recover in case it had bad enough

luck of trying to boot at the same time the TFTP server goes down.

++++ CURRENTLY KNOWN PROBLEMS ++++

The following problems are known with PXELINUX, so far:

+ Requires a TFTP server which supports the "tsize" option.

+ The error recovery routine doesn't work quite right.  For right now,

it just does a hard reset - seems good enough.

+ There may be funnies with memory management.  The PXE spec has no

decent way of telling it to free up all memory and unchain any

interrupts; it allows the base stack to be unloaded, but not the

UNDI driver.

+ There seems to be a problem with sending ACK "storms"; a number of

ACK packets fired off without the proper delay in between.  I

suspect this is a PXE firmware problem, rather than a PXELINUX

problem.

+ We should probably call the UDP receive function in the keyboard

entry loop, so that we answer ARP requests.

+ Boot sectors don't work yet... they probably need auxilliary information

(such as device) to work at all.

If you have additional problems, please contact the SYSLINUX mailing

list (see syslinux.doc for the address.)