Devserv: A Device Controller

SYNOPSIS

devserv [-d <debug_level>] [-s N|n] [-t <ttl>] [-h <video_host>] \

[-v <video_address/video_port/video_ttl>]

DESCRIPTION

devserv is a server for controlling video devices.  The program is meant to be used in conjunction with a client program such as camclnt.  devserv listens on the network for requests from clients and carries out these requests to drive the devices connected to the host computer.  The devices that are currently supported are the Sony EVI-D30, Canon VC-C1, and Canon VC-C3 cameras in conjunction with the Panasonic WJ-MX50 Digital A/V Mixer.

devserv supports requests to pan, tilt, and zoom the cameras,

select the input source to the mixer/switcher, and do picture-in-picture.  Although intended for remote control of video devices, devserv

can be used for local control (in case it is inconvenient to use the

hand-held remote controller unit).

Requests are sent and received by UDP unicast.  Devserv

multicasts description messages when a client requests

them, when any client request has been processed, or if 7.5

minutes have elapsed since the last description was sent.  Any

client may send requests as long as they conform to the "Camera

Remote Control Command Language" (see COMMAND SET below).

OPTIONS

-d  Use debug_level to specify what is displayed on the terminal.

0: nothing is displayed; 1: initialization and termination

status and error messages (if any) are displayed; 2: network events

are displayed; 3: communication between the devices and devserv

are displayed; 4: everything at levels 1, 2, and 3 is displayed.

The default level is 1.

-s  Specify "N" or "n" to suppress turning off the devices when the

program is terminated.

-t  Give an integer for the ttl to specify the time-to-live for

multicast messages.  The default time-to-live is 1.

-h  Use video_host to specify the machine that is the source of

the video.  The default is the local host.  This information

is included in description messages.

-v  The string delimited by "/" specifies the address, port, and

time-to-live used by the video tool for media transmission.

The default value is 0.0.0.0/0/0.  This information is

included in description messages.

OPERATION

Since the device controller is a server, it is intended to be run

like a daemon.  To operate the program, launch it from the DOS

command line.  To quit, type "control-C".

****NOTE:  YOU MUST SET THE "HOME" ENVIRONMENT VARIABLE.  Either in

autoexec.bat or on a DOS command line, before running devserv, make

an entry such as "set HOME=C:\".

Make sure that the main power switches of all attached devices are "on".  On the Sony EVI-D30 and Panasonic WJ-MX50, the main power switches are

on the back of the unit.  Also make sure that the camera control

and pan and tilter units of the Canon VC-C3 camera are turned on.

The Canon VC-C1, Sony EVI-D30, and Panasonic WJ-MX50 allow the power

to be turned on and off via remote control.  However, the Canon VC-C3

does not have this capability.  Therefore, devserv turns on the power

for the Canon VC-C1, Sony EVI-D30, and Panasonic WJ-MX50 at startup

(as long as the main power is turned on) and turns off the power when

you quit the program (unless the -s option is used).

Devserv reads a file at startup to determine the hardware

configuration. This file can also be used to specify video

information and set network addresses if you wish to override the default values. (See CONFIGURATION FILE and NETWORK COMMUNICATION below.)

HARDWARE CONFIGURATION

Since the current version of devserv (v0.4.3) supports RS232 devices,

the RS232 cables for the cameras and video switcher must be connected to

the appropriate serial ports (e.g., COM1 through COM4 on a PC).

Sony EVI-D30 cameras can be daisy-chained with up to 7 cameras per port.

Refer to the user manuals for the specific devices for connections and

configuration.  Note that the Panasonic WJ-MX50 has an "RS422/RS232C

Selection Switch" above the "Editing Control Connector" on its back

panel.  Set the selection switch to "RS232C."  You must also set the

SW701 switch to meet modem requirements.  This is described in the

Operating Instructions Manual for the Panasonic WJ-MX50.

CONFIGURATION FILE

This file is stored in the home directory as $HOME/devconfig (e.g.,

C:\devconfig).  It specifies the hardware configuration, network

parameters, and video information.  Entries are in the form:

keyword value ...

or

keyword label value ...

If a value contains any spaces, it must be enclosed in double

quotes.

The hardware configuration includes the type of devices and what

ports they are connected to, client "handles" for referring to

devices, and internal device addresses.  The serial ports are

COM1, COM2, ...

Each device entry is the form:

device <make>  <model>  <device_number>  <type> \

<unit_number>  <serial port>  <lens-type>

The device number given by <device_number> is used for internal

addressing.  Device addresses should be numbered 1, 2, 3, etc.,

in order from closest to the computer to farthest away.  NOTE:

For the Sony EVI-D30, this number is NOT the camera number on the back of the camera unit. The camera numbers 1, 2, and 3 on the back of the Sony are for use with the hand-held remote commander unit.  THIS NUMBER IS INDEPENDENT OF THE <unit_number>.

The type of lens (standard or wide-angle) on a camera is given by the

lens_type field.  If left blank, the default (standard) is used.

If a wide-angle lens is used, specify "w" or "W".

Since the client program will refer to the devices by the "handle"

assigned by the device controller and this handle consists of the

"type" and "unit_number", you must list these values.  Sample entries

for a PC running Windows 95/NT are:

device Sony EVI-D30 1 camera 1 COM2

device Sony EVI-D30 2 camera 2 COM2

device Panasonic WJ-MX50 1 videoswitch 1 COM1

or

device Canon VC-C3 1 camera 3 COM2 w

device Sony EVI-D30 1 camera 2 COM3

You may use "cam" for "camera" and "vid" for "videoswitch".  The client

handles in the above entries would be: "cam 1", "cam 2", "cam 3", and

"vid 1".

The video information is optional and includes the address, port, and

time-to-live used by the video tool, the host that is the source of the

camera feed, and a text description.  Sample entries are:

video 224.1.2.3/1234/16

video_host  disney.land.com

text "Building XXX, Main Conference Room"

Any line that starts with "#" is treated as a comment.

NETWORK COMMUNICATION

The device controller uses UDP to receive unicast client requests.  The

default UDP port is 5555.  IP multicast is used by the device controller to send status or description messages.  The default multicast address

and port are 224.35.36.37 and 5556.  To override any of these values, make entries in the $HOME/devconfig file.  Entries must be in the

following format:

address udpPort <port>

address multicastAddress <address>

address multicastPort <port>

Example entries are:

address udpPort 8885

address multicastAddress 224.1.2.3

address multicastPort 1234

The standard socket interface is used for this communication.

Users may write their own clients.  They can use the standard

socket interface for network communication between the clients

and the device server or they may use the common communication

library developed under the Collaboratory Interoperability Framework

(CIF) project (if CIF is supported on the current platform).  To accommodate clients that utilize the CIF communication library, devserv also listens on a CIF UDP port to receive unicast client requests and transmits status and description messages via a CIF multicast channel.  The default values for these communication channels are: udp port = 7775, multicast address = 224.35.36.37, multicast port = 7776.  To override these values, make entries in the $HOME/devconfig file in the following format:

address cifUDPPort <port>

address cifUUMAddress <multicast address>

address cifUUMPort <port>

COMMAND SET

The device controller communicates with the client using ASCII text

messages.  While the camclnt program is available, new clients may be

written.  In order to communicate with devserv, messages must follow the

format given by the Camera Remote Control Command Language.

NOTE: A timestamp is given in each request.  If the timestamp is less

than that of the previously accepted command, the command is

ignored (unless it is a request for a description).

BUGS

devserv may not accurately report whether a device is available or not

when there are hardware failures during execution. Status messages are

currently incomplete (e.g., the Camera Remote Control Command Language

specifies that description messages contain a reference to the last

command accepted and this has not yet been implemented).

AUTHOR

Marcia Perry (MPerry@LBL.gov), Lawrence Berkeley National Laboratory, Berkeley, CA