MOUSEBUF.COM     (Mouse Buffer)                       [v0.1: 7/1/94]

============     ==============

Resident program (about 3200 bytes) that gives you mouse based copy

and paste in DOS' textmode.

This program is FreeWare: It might be distributed and used freely.

There exists no warranty, and the author shall not be liable for any

loss or damage due to using this program.

Written in assembly using Borland's great compiler by

Sverre H. Huseby

Bjoelsengt. 17

N-0468 Oslo

Norway

Phone: +47 22 23 05 39

Internet: sverrehu@ifi.uio.no

If you like the program, please do me a small favour: Send me a note

(letter or E-mail) and tell me about it. That's worth more than

a few bucks!

Usage

-----

To be short, hold down the left mouse button and move the mouse to

mark an area. Release the button to copy the marked area to an

internal buffer. Press the right mouse button to insert the contents

of the internal buffer into the keyboard buffer, thus pasting it

into the current application at the cursor position.

To avoid conflicts with other programs, you must (unfortunately ?)

hold down the `hotkey' combination Ctrl and Left Shift while copying

and pasting. This is not required when at the normal DOS command

line, or in any other environment that use INT 21h, function 0Ah

to read a line from the keyboard.

Starting MOUSEBUF

-----------------

The general syntax for starting MOUSEBUF, is as follows:

MOUSEBUF [options]

Options can be zero or more of the following:

-?     Display short help. Any following arguments are ignored.

-b     Set buffer size to the following number of bytes. The

default is -b 1000. Note that it's not possible to change

the size once the program is resident.

-k     Hotkey always needed, that is: Require Ctrl and

Left Shift also on DOS' command line. Can later be turned

off using -k-.

-off   Temporarily disable the program. Can be used for instance

if uninstall is impossible.

-on    Reenable program after -off is used.

-q     Quiet mode, suppress status messages.

-u     Uninstall program if possible. Any resident program

installed after MOUSEBUF must adhere to the AMIS-

standard.

Attention

---------

Be aware of the following:

* A Microsoft compatible mouse driver is required.

* If you hear a beep when you release the left button to end

marking a copy region, it means that too many characters are

marked (the buffer size is exceeded). MOUSEBUF will store as

many characters as will fit.

Other beeps indicate that the mouse driver for some reason

suddenly needs more bytes to save the mouse state than it

did when MOUSEBUF was started.

* If MOUSEBUF can't recognize the current screen mode as text

mode, it won't let you do any copying or pasting.

* Trailing whitespace characters are stripped from all fully

marked lines.

Finally

-------

If you decide to use this program, thanks! I would be happy hearing

from you if you find any bugs, have any suggestions, or just to know

that one more person uses my program.

Sverre.