Quickroute 1.3

--------------

The Professional Solution to PCB/Circuit Design

------------------------------------------------

All software is copyright (c) Dr Ian R Frost 1988,1992

POWERware, 14 Ley Lane, Marple Bridge, Stockport,

SK6 5DD, United Kingdom.

1 Introduction

--------------

Welcome to Quickroute (QR). This powerful package combines a sophisticated

environment with pull down menus, icons, windows and Microsoft(R) mouse

support, with a powerful full-featured PCB or circuit design system which

includes 'rats nest' auto-routing on up to 8 board layers simultaneously.

Quickroute is a SHAREWARE product which means that you are free to

distribute this software by copying the files on the disk intact to another

disk and you are free to try out the software. However, if you wish to

use Quickroute to design PCB's or other graphic designs you MUST register

with POWERware by sending a cheque drawn on a USA bank for $60 to POWERware,

14 Ley Lane, Marple Bridge, Stockport, SK6 5DD, United Kingdom. In return you

will receive a full, good quality A4 manual, the latest version of Quickroute

Professional, a Hewlett Packard plotter driver (HPGL), a 24 pin EPSON style

printer driver, a Laser printer driver and ICONEDIT, a utility for editing

the bit-image symbol type.

PCB designs made with Quickroute can be identified by the decisions it

takes when routing tracks, by the pad outer/inner radii ratio, and in the

way DIL and surface mount pin outs are drawn. Be warned, this product has

taken a great deal of time and effort to develop and POWERware takes the

requirement for registration seriously!

Ensure the continued support of shareware by professional programmers by

registering your use of a product now!

2 Overview

----------

Quickroute includes an immense range of features some of which are:

* Powerful pull-down menu support with ultra-fast icon selection for

frequently used commands.

* Simple but FAST multi-layer auto-routing with rats nest input.

* FAST redraw in any scale from X0.0001 to X10000!

* High quality output to EPSON and IBM generic printers in various qualities

and virtually any scale.

* Powerful symbol manipulation including the ability to grab portions of a

PCB, rotate symbols, edit, etc. Two symbol libraries included (PCB and circuit

diagram).

* Choose from 96 pad sizes/styles, 24 track widths, filled or outlined

circles, ellipses and boxes, various text styles/sizes, arcs, custom

designed 40X40 bit images, any number of library symbols, and range of

instantly available DIL pin outs.

* Use up to 8 layers + 2 silk screen layers. Automatic solder mask

generation.

* Choose objects pictorially from windows!

* Use with virtually any IBM PC/XT/AT/386/486 machine with at least 512K

memory and either a CGA/EGA/VGA or Hercules graphics card. Support for a

Microsoft (R) compatible mouse included.

3 Tutorial

----------

This tutorial covers the basic operation of Quickroute. The full manual is

available when you register with POWERware (see section 1). You will need

this as most of the features are only briefly discussed, and others are not

mentioned at all. If you are going to spend your time developing a design

then it makes sense to register so that you get the most out of this

product.

3.1 Introduction

----------------

In order to run Quickroute you will need and IBM PC/XT/AT or 100% compatible

with at least 512K, a 720K disk drive, and either a CGA/EGA/VGA or Hercules

monochrome graphics card. A Microsoft(R) compatible mouse is also highly

recommended. This should be installed with a suitable driver (see the

manual that came with your mouse).

3.2 Starting Quickroute

-----------------------

If your system has a hard disk drive then you can install Quickroute onto

the drive by inserting the Quickroute disk into drive A and typing INSTALL

at the MS-DOS prompt. Follow the instructions displayed on the screen

carefully. If you don't have a hard disk then simply insert the disk into

drive A and type QR.

Once started a sign on banner will be displayed on your screen followed by

a reminder to register your use of Quickroute. The screen will then clear to

show the main workbench.

The menu line at the top of the screen is accessed by either, pointing the

mouse at the first letter of the menu you wish to pull down and pressing the

right mouse button, or by pressing the first letter of the menu. The menu

will then appear. To select a function again either point the mouse at the

required function and press the right button or press the first letter of

the menu option. You can also use the arrowed cursor keys to move the

highlighted bar up/down the menu. Press return to select the function.

The icons to the right of the screen are accessed by either pointing the

mouse at the icon and pressing the LEFT mouse button or by using the

function keys F1 and F2 to highlight an icon and pressing the space bar.

The status line at the bottom of the screen tells you how much memory you

have got left (the bar), the side you are editing, the object type you are

using and the cursor position. A variety of prompts will also be displayed

here.

3.3 Loading and Editing a PCB

-----------------------------

To get a feel for editing PCB's try loading the demonstration PCB into

Quickroute. To do this pull down the Files menu, and select 'Load Pcb'. Enter

the name 'DEMO.PCB' in the status line and hit return. The PCB will then be

loaded and drawn on the screen in the position it was saved at. To speed up

the screen redraw time, pull down the 'Options' menu and select 'Turbo'.

Alternatively just hit the 't' key. The redraw time will then be considerably

reduced because all objects will be drawn as outlines. To draw a grid again

pull down the 'Options' menu and select 'Grid' or press the 'g' key. Grid and

turbo can be toggled on/off as required.

The scale the demonstration PCB will be loaded in is X1.0. To change the

scale move the cursor to the centre of the screen and press one of the

numeric keys (1-8). The PCB will then be redrawn in a different scale.

Alternatively pull down the 'Options' menu and select zoom. A second menu

will open and the scale can be entered. To enter a custom zoom, say X1.635

simply pull down the 'Edit' menu and select 'Custom zoom'. Enter the required

scale in the status line.

When PCB's are scaled up, the screen becomes a window onto the total design

area. You can move the window around either by moving the mouse pointer to

one of the arrow boxes at the bottom left of the screen and pressing the

LEFT mouse button or by moving the cursor to the edge of the screen using

the normal cursor keys. Alternatively move the cursor to a given area of

the design and press 'p' for pan. The PCB will then be redrawn with the

selected area moved to the centre of the screen.

3.4 Adding Objects onto a PCB

-----------------------------

To place an object onto the design move the cursor to the position where

you wish the object to be placed and press the LEFT mouse button. The

current object type (see the status line) will then be placed. There are six

main object types; pads, tracks, integrated circuits, symbols, icons and

primitives. These are selected by clicking on the appropriate icon to the

right of the screen. Assuming that the top left hand icon is icon 1, and the

bottom left icon is icon 12 then icons allow the placement of respectively,

pads, tracks, integrated circuit pin outs (DIL), symbols, and icons. The icon

containing the question mark is used to change the pad/track/symbol/etc

type or size. The other icons allow the user to add text, circles, arcs,

ellipses and rectangles. Follow the instructions that appear in the status

line to use these.

Once an object type has been selected you can place any number onto the

PCB up to the memory limit.

Example

-------

1. Select the symbol object type by moving the cursor to icon number 4 and

pressing the LEFT mouse button.

2. Change the current symbol type by moving the mouse cursor to icon

number 6 and pressing the LEFT mouse button. You should then see a window

open at the centre of the screen with a page of symbols from the current

library.

3. Move the cursor to one of the symbols shown and press the LEFT button.

The arrows at the bottom left of the window allow you to move backwards and

forwards through the current symbol library a page at a time.

4. Move the cursor to any position on the PCB and press the LEFT mouse

button. The symbol will then be drawn.

3.5  Deleting Objects

---------------------

To delete an object move the mouse cursor to icon 7 ( an eraser symbol) and

press the LEFT mouse button. Then point to the object you wish to delete

(eg. the centre of a pad, the start of a track, the first pin on an ic pin out,

etc) and press the LEFT mouse button. A cross will appear on the object

selected for deletion. You will then be asked to either accept, quit or

continue. Press 'a' or point to the accept box and press the LEFT button to

delete the object. A more powerful block delete facility is described in

section 7.

3.6  Symbol Libraries

---------------------

Quickroute allows you to incorporate any number of symbols from any

number of libraries (up to the limits of computer memory) into your designs.

Symbols can be edited, rotated, added to, deleted, etc, using the commands

available under the 'Symbol' menu. Two sample libraries are provided,

CONFIG.COM, the default PCB symbol starter set, and CIRCUIT.COM, the circuit

diagram symbol set. Try loading in the circuit diagram symbols by pulling

down the 'Symbol' menu and selecting 'Load'. Enter 'CIRCUIT.COM' at the

status line and press return.

3.7  Block Commands

-------------------

Quickroute allows you to select rectangular blocks of your design and

either copy them to other regions of the design, move them to other areas,

delete them, rotate them through a variety of angles, or reflect the blocks

about the x or y axis. To do this pull down the 'Edit' menu and select 'Block'.

Then move the cursor to a given area of the PCB and press return/LEFT

mouse button. Next move the cursor to another region of the PCB so that the

'rubber banded' rectangle contains the region of interest. Press the LEFT

mouse button. Then select Move/Copy/Delete/Rotate/Reflect by pressing the

appropriate key (see the status line). Follow any further instructions in

the status line to complete the operation.

3.8  Autorouting

----------------

The autorouter will automatically attempt to find an unobstructed route

between any two given points. Start and finish positions can be entered

either singly or as a set of rat lines. All routes placed by Quickroute will

contain node angles which are multiples of 45 degrees, will have fewer than

10 nodes, and fit into a rectangle with opposite corners defined by the

start and finish positions. The router is simple but fast.

To use the router, pull down the 'Edit' menu and select either 'Auto-route'

or 'Rats-nest'. You will then be asked to enter one or more start and finish

positions on the PCB. Press the 'Q' key or RIGHT mouse button to start

routing. Note that all sides currently displayed will be used by the auto-

router.

3.9  Printing and Plotting Designs

----------------------------------

The shareware version of Quickroute allows you to print PCB's on either an

IBM or EPSON style 9 pin dot-matrix printer. The printer should be an EPSON

FX compatible, 8" carriage, machine or the equivalent IBM printer.

Registering as a user of this product will entitle you to receive the full

driver pack which includes a Hewlett Packard Graphics Language (HPGL)

driver for pen plotters. Other drivers may also be included (eg. 24 pin dot-

matrix and LaserJet printer support) depending on demand, ie. the number of

people who register!

To print a PCB design, pull down the 'Files' menu and select 'Print PCB'.

Follow the instructions that appear in the status line. Make sure that you

answer 'no' when asked if you wish to change the printer/plotter options, or

you may get unpredictable results.

The default printer/plotter output port is LPT1, this can be changed by

pulling down the 'Files' menu and selecting 'Output'.

3.9  And finally ..

-------------------

This brief manual can only hope to give a taste of the abilities of

Quickroute. Remember that if you wish to use Quickroute to produce PCB's or

any other output you must register by sending a cheque for $60 to

POWERware, 14 Ley Lane, Marple Bridge, Stockport, SK6 5DD, United Kingdom.

In return you will receive the latest version of Quickroute

Professional, a full manual, a full driver pack, and a list of any product

options or updates that are available.

Dr I R Frost, 1992

------------------