STANDARD MICROSYSTEMS CORPORATION

NDIS Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  An overview of the NDIS driver.

o  The files supplied with the NDIS drivers.

o  How to configure the SMC NDIS drivers.

o  The PROTOCOL.INI parameters.

o  Using Memory Managers.

o  Error messages.

OVERVIEW

The NDIS driver is designed to work with all SMC Ultra and EtherCard

Families and the older WD Ethernet LAN Adapters. It conforms to the

Network Driver Interface Specification (NDIS) version 2.0.1 defined

by Microsoft/3Com. The driver has been tested with Microsoft LAN

Manager, 3Com 3+Open, IBM LAN Server, IBM PC LAN Program,

DEC Pathworks, and other NDIS-compatible network operating systems

and protocol stacks.

SUPPLIED FILES

FILE               DESCRIPTION

____               ____________

NDIS.DOC           Installation Guide (this File)

SMC8000.OS2        OS/2 NDIS MAC driver

SMC8000.DOS        DOS NDIS MAC driver

PROTOCOL.INI       PROTOCOL.INI file sample

SMCDOSMC.NIF       NIF file for Micro Channel adapters using DOS

and IBM LAN Server

SMCOS2MC.NIF       NIF file for Micro Channel adapters using OS/2

and IBM LAN Server

SMCDOSAT.NIF       NIF file for AT adapters using DOS and IBM

LAN Server

SMCOS2AT.NIF       NIF file for AT adapters using OS/2 and IBM

LAN Server

SMCDOSJP.NIF       NIF file for Western Digital EtherCard PLUS

adapters using DOS and IBM LAN Server

SMCOS2JP.NIF       NIF file for Western Digital EtherCard PLUS

adapters using OS/2 and IBM LAN Server

SMC.MSG            OS/2 Driver Message File

Follow the instructions in the Installation Guide included with

the adapter to install the adapter card in your PC.

CONFIGURATION

The SMC NDIS drivers may be used to install one or two adapter

cards. When installing a LAN software, select either one or two SMC

Adapters. When the LAN setup is done, examine the CONFIG.SYS

file and the PROTOCOL.INI file.

1. The CONFIG.SYS file should have only one DEVICE= line that

contains the SMC8000.XXX driver for DOS/OS2. For example:

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\SMC8000\SMC8000.DOS

or

DEVICE=C:\LSP\SMC8000.DOS

or

DEVICE=C:\IBMCOM\MACS\SMC8000.OS2

2. The PROTOCOL.INI file should specify one or two SMC8000$ entries.

a. For the first adapter:

[SMC8000_NIF]

DriverName = SMC8000$

IOBase = 0x280

For the second adapter

[SMC80002_NIF]

DriverName = SMC8000$

IOBase = 0x300

Make sure the IOBase value does not conflict with the value

provided for the first adapter, and that it matches

the second adapter setting. Always include an IOBase

statement with the DriverName.

b. Micro Channel adapters -- for the first adapter:

[SMC8000_NIF]

DriverName = SMC8000$

SlotNumber = 3

For the second adapter:

[SMC8000_NIF]

DriverName = SMC8000$

SlotNumber = 4

Make sure the SlotNumber value does not conflict with the

value provided for the first adapter, and that it matches

the second adapter setting. Always include a SlotNumber

statement with the DriverName.

c. A sample entry for an older WD Ethernet adapter that is not

software configurable:

[SMC8000_NIF]

DriverName = SMC8000$

IOBase = 0x300

RAMAddress = 0xCC00

IRQ = 5

3. Modify adapter values so the two adapters do not use the same

values for I/O Base Address, RAM Address, and IRQ. Refer to the

adapter's Installation Guide for information about these

parameters.

PROTOCOL.INI PARAMETERS

Several parameters are typically specified in the SMC8000 section of

the PROTOCOL.INI file. The only parameter that must be specified

is the 'DriverName'. All others have default values supplied by the

driver and can be omitted from the PROTOCOL.INI file if the

installed adapter matches the default configuration. Software-

configurable and Micro Channel adapters ignore the values of IRQ,

IOBase and RAMAddress and instead use the values in the POS

registers set up when the card was installed.

Below is a list of parameters and their definitions commonly found

in the PROTOCOL.INI file. Both default values and possible value

ranges are provided for each parameter.

IOBase       Specifies the base I/O address used by the adapter.

The default is 0x280. This parameter is required for

all ISA adapters.

SlotNumber   Specifies the number of the slot containing the

adapter card in a Micro Channel machine. This

parameter is a decimal number in the range of 1 - 10

and is required for all Micro Channel adapters.

NetAddress   Overrides the factory set network address of the

network adapter. The value of this parameter is a

hexadecimal string of 12 digits between double

quotation marks. The address must be unique among

all network addresses on the network.

The following parameters are required for WD adapters, which are not

software configurable

IRQ          Specifies the interrupt used by the adapter. The

default is 3,

RAMAddress   Specifies the Shared RAM segment address used by the

adapter. The default is 0xD000.

USING MEMORY MANAGERS

After installing your SMC adapter and drivers, use EZStart to view

your adapter's RAM Window.  Then add an "exclude" statement to the

"DEVICE=" line in your CONFIG.SYS file to exclude the RAM Window.

To exclude the RAM Window between 0000 and D3FF using DOS memory

manager EMM386, for example, add:

DEVICE=EMM386.EXE  X=D000-D3FF

If you use a memory manager other than EMM386, refer to the manual

that came with it for information on how to exclude memory.

If you use Microsoft Windows 3.x or Windows for Workgroups, use

EZStart to view your adapter's RAM Window, then exclude it from

the "386 Enhanced" area of your SYSTEM.INI file.  The following

example shows how to exclude the RAM Window between D000 and

D3FF using EMM386:

[386Enh]

EMMEXCLUDE=D000-D3FF

ERROR MESSAGES

"Could not find an adapter [at slot ___] [at I/O base address ___]"

The driver could not find an adapter at the specified location.

"Cannot access adapter RAM properly"

The memory test for adapter memory failed.

"Adapter hardware failure detected (self-test)"

There was an unidentified failure during adapter initialization.

"Adapter hardware failure detected during initialization"

The hardware did not respond correctly to initialization

commands.

"Other system RAM already occupies adapter RAM address space"

There is a RAM conflict with the adapter RAM.

"Other system ROM already occupies adapter RAM address space"

There is a rom conflict with adapter RAM.

"RAM base address chosen for driver does not match adapter

configuration"

The RAM base address must be specified by EZSTART. The definition

in the configuration file must be removed.

"IRQ chosen for driver does not match adapter configuration"

The IRQ value must be specified by EZSTART. The definition in the

configuration file must be removed.

"RAM size chosen for driver does not match adapter configuration"

The RAM size is stored on the adapter and is fixed in size. The

definition in the configuration file must be removed.

"Adapter IRQ higher than 7 not valid in 8-bit slot"

Adapter RAM base address is set to an illegal boundary. Adapter RAM

must start on 200h boundaries.

"I/O base address chosen for driver not valid"

The adapter I/O base address must be set between 200h-ec0h on a 20h

boundary.

"RAM base address chosen for driver not valid (out of range)"

The RAM base address must be between a000h-c000h.

"IRQ chosen for driver not valid (out of range)"

IRQ values must be between 2-15.

"RAM size chosen for driver not valid (out of range)"

RAM size must be either 200h (8k) or 400h (16k).

"Adapter configuration information corrupted; run software setup

program"

The onboard EEROM contents have been corrupted. Please run EZSTART

to verify and correct.

SMC and Standard Microsystems are registered trademarks; and EtherCard

Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite 16C Ultra,

Elite Series, EliteView, EZStart, SuperDisk, and UltraChip are

trademarks of Standard Microsystems Corporation.  Other products

and company names are registered trademarks or trademarks of their

respective holders.