NOVELL NETWARE 386 v3.1 SERVER DRIVER FOR USE WITH

8003/8013 FAMILY LAN ADAPTERS

INSTALLATION GUIDE

The purpose of this document is to briefly explain the

installation of the NetWare 386 LAN drivers that are used

with the 8003/8013 family LAN adapters.

The files in the release diskette are:

INSTALL.DOC     This file.

RELEASE.DOC     Version number and specific release

information.

WDPLUSSV.LAN    Netware 386 StarLAN/Ethernet Server driver.

1.  Refer to the NetWare 386 Installation manual and follow

the instructions to partition the hard disk and to copy

the necessary system files to drive C:.

2.  Copy WDPLUSSV.LAN from this diskette to drive C:.

3.  Follow the "Run SERVER" and the "Load loadable modules"

sections of Novell's Installation Manual to Step (3) -

Load LAN driver module.

load c:wdplussv <Enter>

4.  Answer the prompts for the hardware attributes of each

adapter. Depending on the type of adapter you have, there

are up to four adapter specific configuration parameters

the driver may ask you to supply:

1) I/O Base Address.

2) Interrupt Request (IRQ) Level.

3) RAM Buffer Base Address.

4) Size of RAM Buffer - Enter 200 for  8K bytes

800 for 32K bytes.

5) MCA Bus Slot number (Micro Channel Only).

For the 8003EP, 8003EB, 8003W, 8013EP, 8013W, and 8013EW,

the driver will only prompt you for the I/O Base Address.

For the 8003E, 8003WT, 8003EBT, 8003S, and 8003SH, the

first four parameters listed above are required.

For the 8013EBT adapter, the first three parameters

listed above are required.

For all Micro Channel adapters, only the bus slot number

is required.

If you do not want the prompts to appear, you can include

the required configuration parameters in a command line.

Refer to the NetWare 386 System Administration manual

under LOAD LAN drivers.

For the 8003EP, 8003EB, 8003W, 8013EP, 8013W, and 8013EW:

load c:wdplussv port=280 <Enter>

For the 8003E, 8003WT, 8003EBT, 8003S, and 8003SH:

load c:wdplussv port=280 int=3 mem=d0000:200 <Enter>

For 8013EBT adapters:

load c:wdplussv port=280 int=3 mem=d0000 <Enter>

For all Micro Channel adapters.

load c:wdplussv slot=1 <Enter>

This NetWare 386 server driver when used with StarLAN

adapters does NOT support the old StarLAN packet protocol

used by the Novell StarLAN driver for AT&T StarLAN

adapters. This packet protocol was the default protocol

used on all 8003/8013 family Netware 2.15 workstation

shell drivers prior to driver version 3.07. In order for

these older workstation drivers to function with this

driver, they must be configured to use IEEE 802.3 StarLAN

packets. For instructions on configuring the older

Netware 2.15 workstation drivers for the StarCard PLUS,

refer to the INSTALL.DOC file included with the NetWare

2.15 drivers.

5.  Proceed with Step (4) - Load other loadable modules - to

continue the installation of NetWare 386.

NOTE:

The driver has to be bound to start working (refer to the

section "Bind the protocol to the LAN driver").

This is licensed by Western Digital for use by its customers only.

Western Digital is a registered trademark of Western Digital

Corporation. Other trademarks mentioned herein belong to

their respective companies.

Copyright (c) 1990 Western Digital Corporation

All Rights Reserved.