Here is the layout of a SoundBlaster patch (as

I suppose it to be):

offset:

0-3: "SBI", 0x1a

4-19: voice name

20-28: unused

29: echo delay

30: echo attenuation

31: chorus spread

32: transpose

33: fixed duration

34: extra voice

35: fixed key

36,37: 0x24 (modulator) & 0x25 (carrier)

b7: am vibrato

b6: frequency vibrato

b5: sustain

b4: duration varies with frequency

b3-b0: frequency multiplier

38,39: 0x26 (modulator) & 0x27 (carrier)

b7-b6: amplitude = frequency factor

b5-b0: loudness

40,41: 0x28 (modulator) & 0x29 (carrier)

b7-b4: attack rate

b3-b0: decay rate

42,43: 0x2a (modulator) & 0x2b (carrier)

b7-b4: sustain level

b3-b0: release rate

44,45: 0x2c (modulator) & 0x2d (carrier)

b7-b2: unused

b1-b0: modulator/carrier distortion

46: 0x2e:

b7-b4: unused

b3-b1: phase shift by modulator

b0: 1 = parallel (additive)

0 = frequency modulation

47-: 0x2f-... unused

The layout of an opl3 patch is similar, except that

bytes at offset 0-2 are "4OP", 47-48 are like the

bytes at 36-37 except they are for operators 3 and 4,

49-50 are like 38-39, 51-52 are like 40-41, 53-54 are

like 42-43, and 55-56 are like 44-45.  57 is like

46, except there is no phase shift for operator 3.

58-59 are unused.

53-54

--gl